DEALER:								DATE:	__________________

SALES REP:						JOB NAME:	____________

JOB LOCATION:

This is a step-by-step approach to inspecting and measuring a stage for replacement curtains or repairs and cleaning.  Follow this outline, answer these questions, give these measurements, and we can provide you with a quotation for and a shipment of curtains for exactly what is needed.

GENERAL INFORMATION

1	Is the stage accessible?  ______   is there ramp or access to stage height? _____________________

2	Is there a rear access door or loading dock to access stage?  __________________________

3	Are lights too close to the curtains (should be 18” away from curtains)? ________

4	Are there obstructions to hanging the curtains or hardware such as HVAC ducts, etc. ________?

If so, explain _______________________________________________________________________

5	Is the ceiling exposed or obstructed from view such as with a dropped ceiling? ___________________

Approximate distance from mounting support to drop of ceiling if applicable ______________________

If not obstructed what is the height from the track to the ceiling deck or mounting surface? _________

Are tracks mounted by suspended chain/treaded rod? ______ceiling mount ______wall mount ______

6	Is there access to mounting surface via lift or ladder? _______________________________________

7	Other particular job information needed: __________________________________________________

FRONT CURTAIN VALANCE:   Material is to be:

The existing valance is 	___ inches wide (side-to-side) and 		 feet high (top to bottom).

Valance is to be lined:		YES__	NO Lining Material is to be:			____ or standard.

The existing valance is: 		Tacked to the rear of the stage wall OR 		Tied to a pipe.  It has a 6” fringe: 		Yes		No.  It has a letter(s) reading, in size of		, (color) 	 chenille embroidered on (color) 		 felt, in Style 		.  (The SIZE of the letter is from top to bottom of the letter.  The colors of chenille and felt are selected from the Chenille/Felt color card.  The STYLE of the letter(s) is selected from the catalog showing different styles of lettering available.)

NOTE:  special school emblems can be reproduced in chenille for the valance.  Letters can be enclosed in wreaths.  If you wish either of these accessory items, please attach a drawing or photo of the emblem to be reproduced.  If you want the letters enclosed in an olive wreath, indicate here: 		 Wreath wanted.

SHAPED VALANCES:  Some valances are cut to fit inside curved proscenium openings, or into slanted openings, or into peaked openings.  We can make valances to fit these opening.  If an order is placed, a paper pattern is required for curved valances, and detailed measurements are needed for slanted or peaked openings.  For quoting purposes, however, rough measurements are sufficient.  Show below the SHAPE required for a special valance, and give the WIDTH at the bottom of the valance, the HEIGHT at the lowest point, the HEIGHT at the highest point, plus any other dimensions or information you think might be helpful

DAMAGE TO CURRENT VALANCES

Is the valance damaged in anyway?  ______ dirty, _____ stained, # of tears _______, seam damages______

Are any connectors missing or damaged? ________   number needed _____   type connector? ____________

FRONT CURTAIN:	Material is:			The curtain is lined:		Yes		No   Curtains do not need to be lined unless requested.  If needed, lining is usually black or beige denim.  If another material is desired, name here:

To measure for a front curtain to be installed on an existing track:

The track is usually in two separate pieces, overlapping in the middle.  Measure ONE of these pieces of track, from one end of the piece to the other end of the same piece.  This means you are measuring for one-half the curtain.  This distance is 		.   Count the number of moving carriers on that piece of track, including the master carrier (the carrier at the on-stage end of the track).  There are 		carriers.  Count the number of moving carriers on the other half of the track including the master carrier.  There are 		 carriers on that half of the track.  Are all carriers in working condition?  ________   Are any needed to be replaced? ______, if yes then how many? _________ Carrier type is __________ check the appendix for standard stage curtain carriers to determine most likely type.

Measure from the top of the curtain to the floor.  Tell us below which you measured:

The distance from the top of the curtain to the floor is 			 in inches.  The customer wants 		1”		2”		3” clearance from the bottom of the curtain to the floor.  (2” furnished if customer has no preference.)  Are there trim chains on each carrier? 		Yes	   No

(If there are trim chains on each carrier, you still measure from the top of the curtain to the floor, and we make allowance for starting the new curtains half-way down the trim chain)  If no curtain is hanging measure from the floor to top of track.  Is measure to top of curtain _____ or top of track ______?  OR

The height of the existing curtain from top to bottom is 		.  Fullness __________%

FRONT CURTAIN DAMAGE ISSUES:  Are the curtains damaged in anyway?  ______ dirty, _____ stained, # of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____ if so how many places? ______.  Any tears in top seam?  ______ if so how many places? ________ please take pictures of all issues.

FRONT CURTAIN HARDWARE ISSUES:  Are there any other hardware items that are needed?   Rope tensioner on floor? _____   Does the hardware work properly?  ______, if not is a new track needed? ________

Is the Track suspended by chain? _____   if yes is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track? ________ It is best to have one drop every 5-6 feet for most tracks.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________

Will you need new rope for the track? ___Yes __No.  If Yes, we will need to know the diameter of the existing rope.  Is it 		¼” or 		 3/8”?  Approximate length of rope needed? _______  (times floor to track plus 2 times the length of track)

NOTE THAT ALL CURTAINS ARE SHIPPED WITH EITHER S-HOOKS OR TIELINES, AS INDICATED ON YOUR MEASUREMENT SHEET.

REAR CURTAIN: (also known as the back curtain or cyclorama).  Material is:			The curtain is lined:		Yes		No   Curtains do not need to be lined unless requested.  If needed, lining is usually black or beige denim.  If another material is desired, name here:

Current Status of Front Curtain; dirty? ______ # of stains _____ type of stain if known __________________

Is the stain water damage?  _______ if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____ if so how many places? ______.  Any tears in top seam?  ______ if so how many places? ________ Other issues: __________________________________________________________________________________

To measure for a rear curtain to be installed on an existing track:

The track is usually in two separate pieces, overlapping in the middle.  Measure ONE of these pieces of track, from one end of the piece to the other end of the same piece.  This means you are measuring for one-half the curtain.  This distance is.   Count the number of moving carriers on that piece of track, including the master carrier (the carrier at the on-stage end of the track).  There are 		carriers.  Count the number of moving carriers on the other half of the track including the master carrier.  There are 		 carriers on that half of the track.  Are all carriers in working condition?  ________   Are any needed to be replaced? ______, if yes then how many? _________ Carrier type is __________ check the appendix for standard stage curtain carriers to determine most likely type.

Measure from the top of the curtain to the floor.  Tell us below which you measured:

The distance from the top of the curtain to the floor is 			 in inches.  The customer wants 		1”		2”		3” clearance from the bottom of the curtain to the floor.  (2” furnished if customer has no preference.)  Are there trim chains on each carrier? 		Yes	   No

(If there are trim chains on each carrier, you still measure from the top of the curtain to the floor, and we make allowance for starting the new curtains half-way down the trim chain)  If no curtain is hanging measure from the floor to top of track or pipe if pipe mounted.  Is measure to top of curtain _____ or top of track ______?  OR The height of the existing curtain from top to bottom is 		.   There are how many curtains that form the rear curtain? _____ for each measure the width of the existing curtain at the top of the curtain not the floor.  Curtain 1 ______ wide in inches, Curtain 2 _______” Curtain 3 _________ and Curtain 4 ______ “.  Fullness _____ %

REAR CURTAIN HARDWARE ISSUES:  Are there any other hardware items that are needed?   Rope tensioner on floor? _____   Does the hardware work properly?  ______, if not is a new track needed? ________

Is the Track suspended by chain? _____   if yes is it the proper change?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track? ________ It is best to have one drop every 5-6 feet for most tracks.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________

Will you need new rope for the track? 		Yes		No.  If Yes, we will need to know the diameter of the existing rope.  Is it 		¼” or 		 3/8”?  Feet of Rope needed? ________

NOTE THAT ALL CURTAINS ARE SHIPPED WITH EITHER S-HOOKS OR TIELINES, AS INDICATED ON YOUR MEASUREMENT SHEET.

SIDE CURTAINS, also known as legs, teasers, or wings.  Material:	__________________

These curtains may be on tracks or pipes, and arranged in a variety of ways.  We list a few here.  If we do not describe how this specific stage is arranged, please draw a diagram on paper and send with the dimensions.

A.	One long continuous track with no center overlap on each side of the

stage, from front to back, with one or two or more panels of curtain. .

Measure the track from one end to the other end.  This distance is

There are 		 panels of curtain on this track.  Measure from the

Top of curtain to the floor.  Fullness _____%  This distance is 			. There is a

total of 		 carriers on the entire track.  Do these carriers have

trim chains? 		Yes		No.  Customer wants 	1” or

2” or  		3” of floor clearance from bottom of curtain to floor.

We will allow for half-way down the trim chain in figuring curtain height.  OR The height of the existing curtain is

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: __________________________________________________________________________________

B.	A track on each side of the stage, running from front to back, with a center overlap.

Measure one-half of the track.  This distance is 		.  Count the

number of carriers on this half of the track: 		.  Count the number

of carriers on the other half of the track:		.  Measure from the EYE

OF THE CARRIER to the floor:		.  Fullness ______ %  Are there trim chains on the

carriers? 	Yes		No.

OR:  The height of the existing curtain is

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: __________________________________________________________________________________

C.	Tracks parallel to the front curtain, usually 3 or 4 tracks  on each side of the stage with 1 panel of curtain on each track. The curtains may be fastened to carriers running in a short track, or may be tied to a pipe in a rotating	fixture running in a short  track.

In A Track:	Measure the track from end to end:		.  Measure from the eye of the carrier to the floor:		.  Fullness _____ %  Are there trim chains on this track? 	Yes		No.  There are 	 tracks like this. OR:  measuring the height of the existing curtains, each curtain is to be 	_high.

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: __________________________________________________________________________________

Because there may be several different lengths of track on this layout, we

give additional space following for different lengths:

There are 		tracks that are 		 from end to end, with

curtains that are 		 from the eye of the carrier to the floor.

There are 		 tracks that are 		 from end to end, with

curtains that are 		 from the eye of the carrier to the floor.

D.	One long pipe on each side of the stage, running from front to back, with

one or more curtains tied to it.  Measure the pipe from end to end:

Measure from the BOTTOM of the pipe to the floor:		.  There are

panels of curtain on each pipe.  OR: measuring the height

of the existing curtains, each curtain is to be 		 high.  Fullness? ________ %

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: __________________________________________________________________________________

E.	One or more short pipes on each side of the stage, running parallel to the

front curtain.  Measure each pipe from end to end:		.  Measure

from the bottom of the pipe to the floor:		.  There are 		 pipes

like this.   OR: measuring the height of the existing curtains, each curtain

is to be 		 high.  Fullness? _____ %

Because there may be several different lengths of track on this layout, we

give additional space following for different lengths:

There are 		 pipes that are 		from end to end.  From

bottom of pipe to floor is 		.  OR: Existing curtain is

from top to bottom.  There are 		 pipes like this.

There are 		pipes that are 		from end to end.  From

bottom of pipe to floor is 		.  OR: existing curtain is

from top to bottom.  There are  		 pipes like this.

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: __________________________________________________________________________________

F.	A curved track.  Extend the curtains along the track fully, so that you have a firm idea of the track size.  Measure on the floor with your tape measure from one end of the track, around the curve, across the back, around the other curve, back to the front of the stage.

This total track length is:		.  Is there a center overlap in the center

of the back part of the track?	Yes		No.  Count the total

number of moving carriers on the entire track:		.  How wide is the

track at the back part?		.  There are usually six panels of

curtains on a curved track:  2 on the back, 2 on each side.  Is that the

case here: 		Yes		No.  Measure from the eye of the

carrier to the floor: 		.  Are there trim chains on the carriers ?

Yes		No.  OR The height of the existing curtains from top to bottom is

Current Status of Side Curtain;  Dirty? ______  # of stains _____  type of stain if known __________________

Is the stain water damage?  _______  if water damage is there a leak nearby? _________________________

(please take a picture of all issues noted) # of tears in curtain  _______ number of seams damaged ________# of tears _______, seam damages______, chain/weights/pipe coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  Other issues: ________________________________________________________________________________________

SIDE CURTAIN HARDWARE ISSUES:

TRACK ONE-Are there any other hardware items that are needed?   Rope tensioner on floor?_____   Does the hardware work properly?  ______, if not is a new track needed? ________

Is the Track suspended by chain? _____   if yes is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track? ________  It is best to have one drop every 5-6 feet for most tracks.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________

Will you need new rope for the track? 		Yes		No.  If Yes, we will need to know the diameter of the existing rope.  Is it 		¼” or 		 3/8”?  How many feet?  _____  Is track walk draw or rope operated?  _______  NOTE ABOVE FOR EACH PIECE OF TRACK.

TRACK TWO-Are there any other hardware items that are needed?   Rope tensioner on floor?_____   Does the hardware work properly?  ______, if not is a new track needed? ________

Is the Track suspended by chain? _____   if yes is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track? ________  It is best to have one drop every 5-6 feet for most tracks.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________

Will you need new rope for the track? 		Feet?		No.  If Yes, we will need to know the diameter of the existing rope.  Is it 		¼” or 		 3/8”?  Is track walk draw or rope operated?  _______  NOTE ABOVE FOR EACH PIECE OF TRACK.

REPEAT INFORMATION FOR ADDITIONAL TRACKS AS NEEDED

SKY DROPS OR SCENERY CURTAINS:

Sky Drops are used to represent the sky, and are always a light sky blue

in color.    Scenery curtains are either a plain backdrop or could be in grey. black, white or natural.  These drops have no fullness (no pleats).  They can hang from carriers in a track, or be tied to a pipe.  You need to measure the existing curtain if possible.

This curtain is 		wide and 		high.  The seams run from

side to side or 	top to bottom or the curtain is SEAMLESS____

There are three ways the bottom of this curtain can be made.  Check below	which apply:

There is a piece of pipe inside the bottom hem.  This piece

of pipe is 		“ in diameter.

There is a piece of pipe tied onto the bottom hem with tie-

lines through grommets in the bottom hem.

The curtain has a weight inside the bottom hem so it can be

stacked at one end of the track.

SKY DROP CURTAIN HARDWARE ISSUES:   Are there any other hardware items that are needed?   Rope tensioner on floor?_____   Does the hardware work properly?  ______, if not is a new track/pipe needed? ________  if so explain what is needed ________________________________________________________

Is the Track/pipe suspended by chain? _____   if yes, is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track/pipe? ________  It is best to have one drop every 5-6 feet (8 feet for pipe) for most tracks/pipes.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________ Any other issues? ____________________________________________________

NOTE ANY DAMAGE TO SKY DROP BELOW

SCRIM CURTAIN:		This is a curtain that looks something like mosquito

netting.  Usually there is only one of these on well-equipped stages.  This

curtain is always in one piece, flat (no pleats-% fullness) and there are no seams in

the curtain.  It may hang from a track or from a pipe.  You should measure

the existing curtain if possible.

This curtain is 		wide and 		high.  The color is :

Natural (very light beige) 		White  	Black		Blue

There are three ways the bottom of the curtain can be made.  Check which apply?

There is a piece of pipe inside the bottom hem, and this piece of

pipe is 	“ in diameter.

There is a piece of pipe tied onto the bottom hem with tielines.

The bottom has weights inside the hem so it can be stacked at one

end of the track.

SCRIM CURTAIN HARDWARE ISSUES:   Are there any other hardware items that are needed?   Rope tensioner on floor?_____   Does the hardware work properly?  ______, if not is a new track/pipe needed? ________  if so explain what is needed ________________________________________________________

Is the Track/pipe suspended by chain? _____   if yes, is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track/pipe? ________  It is best to have one drop every 5-6 feet (8 feet for pipe) for most tracks/pipes.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________ Any other issues? ____________________________________________________

NOTE ANY DAMAGE TO SCRIM  BELOW

MID-TRAVELER CURTAIN:	Material is:			The curtain is lined:		Yes		No   Curtains do not need to be lined unless requested.  If needed, lining is usually black or beige denim.  If another material is desired, name here:

To measure for a front curtain to be installed on an existing track:

The track is usually in two separate pieces, overlapping in the middle.  Measure ONE of these pieces of track, from one end of the piece to the other end of the same piece.  This means you are measuring for one-half the curtain.  This distance is:		.   Count the number of moving carriers on that piece of track, including the master carrier (the carrier at the on-stage end of the track).  There are 		carriers.  Count the number of moving carriers on the other half of the track including the master carrier.  There are 		 carriers on that half of the track.  Are all carriers in working condition?  ________   Are any needed to be replaced? ______, if yes then how many? _________  Carrier type is __________  check the appendix for standard stage curtain carriers to determine most likely type.

Measure from the top of the curtain to the floor.  Tell us below which you measured:

The distance from the top of the curtain to the floor is 			 in inches.  The customer wants 		1”		2”		3” clearance from the bottom of the curtain to the floor.  (2” furnished if customer has no preference.)  Are there trim chains on each carrier? 		Yes	   No

(If there are trim chains on each carrier,  you still measure from the top of the curtain to the floor, and we make allowance for starting the new curtains half-way down the trim chain)  If no curtain is hanging measure from the floor to top of track.  Is measure to top of curtain _____ or top of track ______?  OR

The height of the existing curtain from top to bottom is 		.  Fullness is _____ %

MID-TRAVELER CURTAIN DAMAGE ISSUES:  Are  the curtains damaged in anyway?  ______  dirty,  _____ stained, # of tears _______, seam damages______, chain/weights coming out of the bottom seam? _____  if so how many places? ______.  Any tears in top seam?  ______  if so how many places? ________  please take pictures of all issues.

MID-TRAVELER HARDWARE ISSUES:  Are there any other hardware items that are needed?   Rope tensioner on floor?_____   Does the hardware work properly?  ______, if not is a new track needed? ________

Is the Track suspended by chain? _____   if yes is it the proper chain?  ______ Chain needs to be heavy duty and not light weight such as dog chain.  Is the chain secured to the track by locking quicklinks?  _______

How many drops of chain support the track? ________  It is best to have one drop every 5-6 feet for most tracks.   If track is mounted other way note mount _____ wall, ______ ceiling, Other, ______ how __________

Will you need new rope for the track? 		Feet?		No.  If Yes, we will need to know the diameter of the existing rope.  Is it 		¼” or 		 3/8”?

NOTE THAT ALL CURTAINS ARE SHIPPED WITH EITHER S-HOOKS OR TIELINES, AS INDICATED ON YOUR MEASUREMENT SHEET.

BORDERS (also known as sky borders, or sometimes teasers, or

sometimes valances.

Material is to be

Borders are 		tied to a pipe or 		tacked to a board.

Measure the pipe to which the borders are tied or the board to which they

are tacked.  Measure from one end to the other.  Measure from the top of

the existing border to the bottom.  Note that borders may be different sizes

or all one size.

There are 		border(s) each 		wide and 		high. Fullness ___%

There are 		border(s) each 		wide and 		 high. Fullness ___%

There are 		 border(s) each 		 wide and 		 high. Fullness ___%

DAMAGE TO CURRENT BORDERS

BORDER 1

Is the valance damaged in anyway?  ______  dirty,  _____ stained, # of tears _______, seam damages______

Are any connectors missing or damaged? ________   number needed _____   type connector? ____________

BORDER 2

Is the valance damaged in anyway?  ______  dirty,  _____ stained, # of tears _______, seam damages______

Are any connectors missing or damaged? ________   number needed _____   type connector? ____________

BORDER 3

Is the valance damaged in anyway?  ______  dirty,  _____ stained, # of tears _______, seam damages______

Are any connectors missing or damaged? ________   number needed _____   type connector? ____________

ACCESSORIES:

CURTAIN COVERS:  these are covers that protect curtains when they are

stacked for storage at the end of the track.  Used most often for front

curtains, sky drops and scrims.  The covers are wrap-around style, made

of heavy vinyl covered polyester, held together with Velcro-brand tape,

and with a batten at each end to keep them upright.  The standard size is

8’0” high, with width made according to the curtain to be protected.

I need 		pair of covers.  The curtains the covers are to protect

are the								.

HEADERS:  These are small curtains that match the side or back curtains.

They are used to fill in the space above stage flats and properties.  This

space occurs when curtains are pulled aside in order to set a door or

window flat in between the curtains.  The HEADER curtain is attached to

the track or pipe above the flat.  Single headers are 4’0 wide and double

headers are 6’0 wide.  Their height is from the track or pipe to 6’6” from

the floor.

This job requires 		single headers and/or 	 double headers.

.

REPLACEMENT HARDWARE:

You may need to order replacement carriers, or new floor tension pulleys,

or new cord.  We must know the kind of track existing in order to send the

correct parts.  Look at page full size end sections of the most commonly

used tracks to identify what kind of track is existing, and list needed parts below:

Section “A” is model #280 draw track or #284 walk draw track.  Can not be curved

Section “B” is #170 track , if rope operated, or #173 walk-draw track

Section “C” is #140 track if roped, or #142 if walk-draw.  Can be curved.

Section “D” is #132 walk-draw track.  Can be curved